# 5. Automating Application Delivery with F5 LTM and Ansible

In this chapter, we will outline how to automate F5 BIG-IP platforms running as load balancers or LTM appliances. We will explore how to interact with F5 LTM nodes using Ansible and how to onboard these devices and accelerate application deployment hosed by these devices using various Ansible modules. We will base our illustration based on the below sample network diagram of a single F5 LTM nodes connected to DC Switches.

The main recipes covered in this chapter is shown below

- Building Ansible Network Inventory.

- Connecting and User Authentication to BIG-IP.

- Configuring Basic System settings on BIG-IP.

- Configuring Interfaces on BIG-IP.

- Configuring VLANs and Self-IPs on BIG-IP.

- Configuring Static Routes on BIG-IP.

- Configuring Nodes on BIG-IP.

- Configuring Load Balancing Pool on BIG-IP.

- Configuring Virtual Servers on BIG-IP.

- Retrieving Operational Data from BIG-IP.

# Technical requirements

All the code used in the recipes in this chapter can be found on the below GitHub Repo

https://github.com/PacktPublishing/Network-Automation-Cookbook/tree/master/ch5_f5

Below are the Software releases that this chapter is based on

- Ansible Machine Running Ubuntu 16.04

- Ansible 2.8

- F5 BIG-IP running BIG-IP 13.1.1 Build 0.0.4 Final

# Building Network Inventory

In this recipe, we will outline how to build and structure our Ansible Inventory to describe our simple F5 BIG-IP nodes. Building an Ansible inventory is a mandatory step in order to tell Ansible how to connect to the managed devices.

## Getting Ready

We create a new folder that will host all the files that we will create in this chapter. The new folder is named ch5_f5_netops.

## How to do it..

- Inside the new folder (ch5_f5_netops) we create hosts file with the below content

$ cat hosts

[ltm]

ltm01    ansible_host=172.20.1.34

- Create ansible.cfg file as shown below

$ cat ansible.cfg
[defaults]
inventory=./hosts
retry_files_enabled=False
gathering=explicit
host_key_checking=False

## How it works..

Since we have a single LTM nodes our hosts file is very simple we create a single group (called ltm) and we specify a single node in it which is ltm01 and we specify the management IP addresses for the nodes as the ansible_host.

The management port on the BIG-IP must have this IP address configured and IP connectivity between the Ansible control machine and the BIG-IP node on this management port is established.

Finally, we create the ansible.cfg file and configure it to point to our hosts file to be used as ansible inventory file and we disable the setup module which is not needed when running ansible against network nodes.

# Connecting and Authenticating to BIG-IP Devices

In this recipe, we will outline how to connect to BIG-IP Nodes from Ansible via the REST API exposed by BIG-IP in order to start managing the devices from Ansible. We are going to use Username and passwords to authenticate to the BIG-IP Node in our topology.

## Getting Ready

In order to follow along with this recipe, an ansible inventory file should be constructed as per the previous recipe, also IP reachability between the Ansible Control machine and all the devices in the network must be established.

## How to do it..

- Inside the folder ch5_f5_netops create the folder group_vars.

- Inside the group_vars folder create the YAML file all.yml with the below contents

conn_parameters:

user: admin

password: admin

server: "{{ ansible_host }}"

server_port: 443

validate_certs: no

admin_passwd: NewP@sswd

users:

- name: ansible

passwd: ansible123

role: all:admin

state: present

- Create a new playbook with the name pb_f5_onboard.yml with the below contents

- name: Onboarding a New LTM

hosts: ltm01

connection: local

tasks:

- name: "P1T1: Create new Users"

bigip_user:

username_credential: "{{ item.name }}"

password_credential: "{{ item.passwd }}"

partition_access: "{{ item.role }}"

state: "{{ item.state | default('present')}}"

provider: "{{ conn_parameters }}"

loop: "{{ users }}"

    - name: "P1T1: Update admin Password"

bigip_user:

username_credential: admin

password_credential: "{{ admin_passwd }}"

state: present

provider: "{{ conn_parameters }}"

## How it works..

In order to establish a REST API connection with the BIG-IP system we need to provide some parameters in order for Ansible to initiate and establish the REST connection with the BIG-IP node. These parameters include the following

- Username/password to authenticate with the BIG-IP REST API

- IP address  and port over which the REST API is deployed on  BIG-IP node.

- Whether we validate the certificate for the BIG-IP node negotiated over the HTTPs session.

All these parameters we include them in a dictionary called conn_parameters which we include it in the all.yml file under the group_vars directory so as to be applied on any BIG-IP node.

By default, a new LTM device comes with the admin/admin default username and password for GUI and REST API access. We use these credentials as the user and password variables inside the conn_parameters dictionary and we specify the ansible_host variable as the IP address over which the REST API can be established over port 443 and we disable certificate validation since certificate on the BIG-IP node is self-signed.

We create a new variable called users which holds all the new users that we want to configure on our LTM along with their role/privilege and in this case we want to provide administrative privilege for the user ansible across all the partitions on the LTM node.

We create a new playbook for onboarding a new LTM nodes and we specify in the first task we create the new users using bigip_user module and we provide the parameters to establish the REST API to the provider attribute and we loop over all the users in our users variable to provision them.

The second task is also using the bigip_user in order to update the default admin profile on the LTM and change this default password to a new password specified in the admin_passwd variable.

On the playbook level we are setting the connection to local since we are going to establish REST API connection from the ansible control machine as we want to prevent ansible from SSH to the LTM node.

The below screenshots outline the new Ansible user created on the BIG-IP node

The below screenshot outline the details of the ansible user created using the playbook.

Note:
We are using plain text password for simplicity however plain text password shouldn’t never be used and nsible vault should be used to secure the password

## There is More..

After adding the new ansible user we update the conn_parameters dictionary with the new user that we have created in order to start manage the LTM nodes using this user as shown below

$ cat group_vars/all.yml
conn_parameters:

user: ansible

password: ansible123

server: "{{ ansible_host }}"

server_port: 443

validate_certs: no

< --  Output Omitted for brevity -->

# Configuring Generic System Options on BIG-IP Devices

In this recipe, we will outline how to configure some basic system options like hostname, DNS and NTP on BIG-IP nodes. . We will understand how to setup all these system level parameters using the various Ansible modules available.

## Getting Ready

To follow along with this recipe, an ansible inventory is assumed to be already setup and IP connectivity between Ansible and the BIG-IP nodes is already established  with the correct user credentials.

## How to do it..

- Update the all.yml file inside the group_vars folder as shown below

$ cat  group_vars/all.yml

< --  Output Omitted for brevity -->

domain: lab.net

nms_servers:

- 172.20.1.250

- Create a new folder called tasks and create the file f5_system.yml with the below contents

$ cat tasks/f5_system.yml

---
- name: "Setup BIG-IP Hostname"

bigip_hostname:

hostname: "{{ inventory_hostname }}.{{ domain }}"

provider: "{{ conn_parameters }}"

- name: "Setup BIG-IP DNS Servers"

bigip_device_dns:

ip_version: '4'

name_servers: "{{ nms_servers }}"

provider: "{{ conn_parameters }}"

- name: "Setup BIG-IP NTP Servers"

bigip_device_ntp:

ntp_servers: "{{ nms_servers }}"

provider: "{{ conn_parameters }}"

- In the pb_f5_onboard.yml file, add the below highlighted tasks

$ cat pb_f5_onboard.yml

< --  Output Omitted for brevity -->

- name: "P1T3: Configure System Parameters"

import_tasks: "tasks/f5_system.yml"

tags: system

## How it works..

In order to configure the various system parameters on BIG-IP nodes we use a separate module for each task. We group all these tasks in a single file  called  f5_system.yml  under the tasks folder and inside this file we use three separate tasks/modules as shown below

- bigip_hostname to setup the hostname

- bigip_device_dns to setup the DNS server that BIG-IP node will use

- bigip_device_ntp to setup the NTP servers on the BIG-IP node.

All these modules take the conn_parameters dictionary to correctly setup how to communicate to the REST API of the BIG-IP node.

In this sample topology we use a single server as the DNS and NTP and we describe it using the nms_servers variable in the all.yml file to apply to all our nodes in our ansible inventory.

In order to configure the hostname we need to supply a FQDN for the device so we configure our domain again under the all.yml file and use it in conjunction with the device name to setup its hostname.

After running this playbook we can see that all the configuration is applied to the BIG-IP node, the below screenshot outline the hostname is correctly provisioned.

The NTP configuration is deployed correctly as per the below screenshot

The DNS is configured correctly as per the below screenshot

# Configuring Interface and Trunks on BIG-IP Devices

In this recipe we will outline how to setup trunks on BIG-IP Devices. Trunk ports on BIG-IP nodes are to provide increased redundancy for the device by combining multiple interface into a single logical interface and it is very similar to port-channels in traditional network vendors.

## Getting Ready

To follow along with this recipe, an ansible inventory is assumed to be already setup and IP connectivity between Ansible and the BIG-IP nodes is already established with the correct user credentials.

## How to do it..

- Create the host_vars folder and create a ltm01.yml file with the below contents

$ host_vars/ltm01.yml
----
phy_interfaces:

- 1.1

- 1.2

trunks:

- name: po1

members: "{{ phy_interfaces }}"

- Under the tasks folder add a new file called f5_interfaces.yml with the below contents

$ cat tasks/f5_interfaces.yml

---

- name: Create a Port channel on BIG-IP

bigip_trunk:

name: "{{ item.name}}"

interfaces: "{{ item.members }}"

link_selection_policy: maximum-bandwidth

frame_distribution_hash: destination-mac

lacp_enabled: no

provider: "{{ conn_parameters }}"

state: present

loop: "{{ trunks }}"

- Update the playbook pb_f5_onboard.yml with the below new task

$ cat pb_f5_onboard.yml

< -- Output Omitted for bevirty -->

- name: "P1T4: Configure Interfaces"

import_tasks: "tasks/f5_interfaces.yml"

tags: intfs

## How it works..

We define the host specific data for the LTM device under the host_vars in a file called ltm01.yml. In this file we define the physical interface on the LTM node under the variable phy_interfaces and we define another variable which is trunks in order to define the trunks available on the define. In the trunk variable we reference the phy_interfaces in order to limit data duplication.

On the f5_interfaces task file we add a new task using the bigip_trunk module to provision the required trunks on the BIG-IP node and we loop over the trunks data structure to provision all the required trunk ports.

After running the playbook we can see that the required trunk interfaces are provisioned as shown below

## See Also..

For more options regarding how to deploy trunk ports on the BIG-IP nodes like enabling LACP and how to adjust multiple LACP parameters on the trunk ports please refer to the blow URL.

https://docs.ansible.com/ansible/latest/modules/bigip_device_ntp_module.html#bigip-trunk-module

# Configuring VLANs and IPs on BIG-IP Devices

In this recipe, we will outline how to configure VLANs on BIG-IP nodes, VLANs on the BIG-IP nodes are fundamental for traffic separation for the different applications hosted by the BIG-IP LTM nodes and they are fundamental to designate external (Internet facing) and internal (server facing) domains. We also outline how to assign IP address on the VLAN interfaces are we provision.

## Getting Ready

To follow along with this recipe, an ansible inventory is assumed to be already setup and IP connectivity between Ansible and the BIG-IP nodes is already established with the correct user credentials. Also since all the VLANs in this setup will be deployed on trunk ports we need to have the trunk ports already provisioned as per the previous recipe.

## How to do it..

- Update the ltm01.yml file under the host_vars with the below contents

$ cat host_vars/ltm01.yml

< -- Output Omitted for bevirty -->

vlans:

- vlan: 100

description: Extrnal VLAN (Internet)

ip: 10.1.100.254/24

tagged_intf: po1

- vlan: 10

description: Server VLAN10 (Internal)

ip: 10.1.10.254/24

tagged_intf: po1

- Update the  f5_interfaces.yml file under the tasks folder with the task to provision VLANs.

$ cat tasks/f5_interfaces.yml

< -- Output Omitted for bevirty -->

- name: Create VLANs on BIG-IP

bigip_vlan:

tagged_interfaces: "{{ item.tagged_intf }}"

name: "VL{{item.vlan}}"

description: "{{ item.description }}"

tag: "{{item.vlan}}"

provider: "{{ conn_parameters }}"

state: present

loop: "{{ vlans }}"

- Update the  f5_interfaces.yml file under the tasks folder with the task to provision Self-IP on the respective VLANs.

$ cat tasks/f5_interfaces.yml

< -- Output Omitted for bevirty -->

- name: Provision IP addresses on BIG-IP

bigip_selfip:

address: "{{ item.ip | ipv4('address') }}"

name: "VL{{ item.vlan }}"

netmask: "{{ item.ip | ipv4('netmask') }}"

vlan: "VL{{ item.vlan }}"

provider: "{{ conn_parameters }}"

state: present

loop: "{{ vlans }}"

## How it works..

We add the vlans data structure in ltm01.yml under the host_vars to declare all the VLANs that we need to provision on the LTM node along with the IP addresses associated with this VLAN.

We update the f5_interfaces.yml file with a task using bigip_vlan module to provision the vlans on the BIG-IP node and we loop over the vlans data structure to extract all the required parameters to setup the needed VLANs. Next, we add another task using bigip_selfip ansible module to deploy the IP addresses on the VLANs.

After running the playbook again, we can see the VLANs and Self-IPs on the BIG-IP node as shown below

The Correct IP address is configured correctly on the VLAN interface as per the below screenshot

## See Also..

For more options regarding how to deploy VLANs and self-IPs on the BIG-IP nodes like please refer to the blow URL.

bigip-vlan

https://docs.ansible.com/ansible/latest/modules/bigip_selfip_module.html#bigip-vlan-module

bigip-selfip

https://docs.ansible.com/ansible/latest/modules/bigip_selfip_module.html#bigip-selfip-module

# Configuring Static Routes on BIG-IP Devices

After deploying the VLANs and IP addresses on the BIG-IP, we need to configure routing on the BIG-IP nodes in order to reach external destination. We use static routes in our topololgy in order to provision the required routing on the LTM node. In this recipe we will outline how to configure Static routes on BIG-IP.

## Getting Ready

To follow along with this recipe, an ansible inventory is assumed to be already setup and IP connectivity between Ansible and the BIG-IP nodes is already established with the correct user credentials. Further, we need to deploy the VLANs and IP addresses in the BIG-IP node as per the previous recipe.

## How to do it..

- Update the ltm01.yml file under the host_vars with the below contents

$ cat host_vars/ltm01.yml

< -- Output Omitted for bevirty -->

routes:

- dst: 0.0.0.0/0

gw: 10.1.100.1

name: default_route

- Update the  pb_f5_onboard.yml  with the below task

$ cat pb_f5_onboard.yml

< -- Output Omitted for bevirty -->

- name: "P1T5: Setup External Routing"

bigip_static_route:

destination: "{{ item.dst.split('/')[0] }}"

netmask: "{{item.dst | ipv4('prefix')}}"

gateway_address: "{{ item.gw }}"

name: "{{ item.name }}"

provider: "{{ conn_parameters }}"

loop: "{{ routes }}"

tags: routing

## How it works..

We add the routes data structure in ltm01.yml under the host_vars to declare all the static routes that needs to be provisioned on the LTM node.

We update the playbook pb_f5_onboard-.yml with a task to provision the static routes using the bigip_static_route module and we loop over the routes data structure to provision all the needed routes on the device.

After running the playbook again, we can see the correct static routes as shown below

# Deploying Nodes on BIG-IP Devices

In order to start deploying Application on the BIG-IP LTM nodes in order to start load balancing the application traffic across multiple application instances, we need to define the instances that will be carrying the application in order to add them to the correct load balancing pool. In BIG-IP these instances are called nodes and it identify each server with a unique IP address.  In this recipe we are going to start deploying a new Application (Web Server) on the BIG-IP and we will provision the nodes which are carrying this service using Ansible.

## Getting Ready

Basic Setup for the BIG-IP should be already completed as per the previous recipes and the correct VLANs to reach these nodes (physical servers) must be deployed.

## How to do it..

- Create a new YAML file called web_app.yml with the below content

---

vip: 10.1.100.10

vip_port: 443

endpoint: dev.internet.net

pool_name: dev_web_app

pool_members:

- ip: 10.1.10.10

name: "dev01.internal.net"

port: 443

- ip: 10.1.10.11

name: "dev01.internal.net"

port: 443

- Create a new Ansible playbook called pb_f5_deploy_app.yml with the below contents

---

- name: Deploying a New App on BIG-IP

hosts: ltm01

connection: local

vars_file: web_app.yml

tasks:

- name: "Create Nodes on BIG-IP"

bigip_node:

address: "{{ item.ip }}"

name: "{{ item.name }}"

provider: "{{ conn_parameters }}"

state: present

loop: "{{ pool_members }}"

## How it works..

We define all the parameters for our new Web application that should be hosted on the BIG-IP LTM device in a YAML file called web_app.yaml. In this file we include a parameter which is the pool_members to outline the web servers which will house the application. We use this parameter to create the nodes on the BIG-IP LTM.

We create a new playbook for application deployment called pb_f5_deploy_app.yml and we include the web_app.yml file so as to have access to all the parameters defined for this app. We create a new task using the bigip_node module to provision a new node on the BIG-IP Node and we loop through the pool_members parameter derived from the web_app.yml file to provision all the required nodes on the BIG-IP node. In order to connect to the BIG-IP node we use the same previous provider attribute with the conn_parameters parameter defined in the all.yml in order to establish the REST connection with the BIG-IP.

Running this playbook, we create the all the needed nodes as shown below

# Configuring LB Pools on BIG-IP Devices

After creating a Node on the BIG-IP, we need to create a load balancing pool for the application that we are deploying and assign pool members from the nodes that we have created into this pool. In this recipe we will outline how to provision Load balancing pools on the BIG-IP nodes and how to assign members to the load balancing pool.

## Getting Ready

This recipe assumes that all the previous recipes has been implemented and the nodes on the BIG-IP is already provisioned as per the previous receipe.

## How to do it..

- Update the playbook pb_f5_deploy_app.yml with the below task to create a new pool

---

- name: Deploying a New App on BIG-IP

hosts: ltm01

connection: local

vars_file: web_app.yml

tasks:

< -- Output Omitted for brevity -->

    - name: Create New LB Pool

bigip_pool:

name: "POOL_{{ website }}_{{ vip_port }}"

lb_method: round-robin

state: present

provider: "{{ conn_parameters }}"

- Update the playbook pb_f5_deploy_app.yml with the below task to assign pool members to the new created pool

---

- name: Deploying a New App on BIG-IP

hosts: ltm01

connection: local

vars_file: web_app.yml

tasks:

< -- Output Omitted for brevity -->

- name: Add Members to the Pool

bigip_pool_member:

pool: "POOL_{{ website }}_{{ vip_port }}"

host: "{{ item.ip }}"

name: "{{ item.name }}"

port: "{{ item.port }}"

description: "Web Server for {{ website }}"

provider: "{{ conn_parameters }}"

loop: "{{ pool_members }}"

## How it works..

In this recipe we create a load balancing pool on the BIG-IP system using the bigip_pool module and we specify the load balancing technique that should be used on this pool and in this example, we are using the round-robin technique. We create the pool name using the different parameters extracted from the web_app.yml file (mainly the website and vip_port) .

Next, we assign the pool members to this newly created pool using the bigip_pool_member module and looping through all the pool_memebers defined again in the web_app.yml.

We can see that all these procedures create a consistant method for defining the pool names as well as assigning the required pool members to the correct pool member. All the information is retrieved from a single definition file that describe and outline how the service should be deployed.

Running these two tasks we will see that the pool is correctly created as shown below with the correct pool members

## See Also..

In this recipe we outlined the basic use for the ansible modules to provision LB pools on the BIG-IP nodes however there is more options available for these modules like the specifying the load balancing ratio for each member as well as attaching monitors for the overall pool. Please consult the below URLs for more options

Bigip_pool
https://docs.ansible.com/ansible/latest/modules/bigip_pool_module.html#bigip-pool-module

Bigip_pool_member
https://docs.ansible.com/ansible/latest/modules/bigip_pool_member_module.html#bigip-pool-member-module

# Creating Virtual Servers on BIG-IP

The last part in deploying an application on the BIG-IP LTM for load balancing is configuring the virtual server on the BIG-IP LTM node and creating a VIP on the BIG-IP node for this virtual server. In this recipe we outline how to deploy the virtual server using Ansible

## Getting Ready

This recipe assumes that all the previous recipes are completed and a load balancing pool and pool members are already configured.

## How to do it..

- Update the playbook  pb_f5_deploy_app.yml with the below task

---

- name: Deploying a New App on BIG-IP

hosts: ltm01

connection: local

vars_file: web_app.yml

tasks:

< -- Output Omitted for brevity -->

    - name: Create Virtual Server

bigip_virtual_server:

name: "{{ website }}_{{ vip_port }}_VS"

destination: "{{ vip }}"

port: "{{ vip_port}}"

pool: "POOL_{{ website }}_{{ vip_port }}"

description: "VIP for {{ website }}"

profiles:

- http

- name: clientssl

context: client-side

- name: serverssl

context: server-side

state: present

provider: "{{ conn_parameters }}"

## How it works..

We use the bigip_virtual_server module to provision the required virtual server on the BIG-IP node specifying the parameters defined from the web_app.yml file. We also provision define the profiles that need to be applied to the new created virtual server and which like HTTP and SSL profiles. These profiles are already created by default on the BIG-IP node and in case if we need to create custom profiles we need to create it in a separate task using the appropriate ansible module.

Running this last task we can see that the Virtual Server is created as shown below

With this last task we have created a functional service VIP on the LTM node in order to start processing HTTP requested for our new website and to load balance the traffic across all the instances in the load balancing group.

## See Also..

In this recipe we discussed the basic use for the ansible module to provision virtual servers on the BIG-IP nodes , however there is more options available in order to tweak the configuration for the virtual server that need to be deployed. Also there are more Ansible modules that let’s you create profiles that you can use to attach to the virtual server . below are some links for this modules from more information.

Ansible bigip_virtual_server Module
https://docs.ansible.com/ansible/latest/modules/bigip_virtual_server_module.html#bigip-virtual-server-module

Ansible bigip_profile_http Module
https://docs.ansible.com/ansible/latest/modules/bigip_profile_http_module.html#bigip-profile-http-module

Ansible bigip_profile_client_ssl Module
https://docs.ansible.com/ansible/latest/modules/bigip_profile_client_ssl_module.html#bigip-profile-client-ssl-module

Ansible bigip_profile_server_ssl Module
https://docs.ansible.com/ansible/latest/modules/bigip_profile_server_ssl_module.html#bigip-profile-server-ssl-module

# Retrieving Operational data from BIG-IP Nodes

In this recipe we outline how to retrieve operational data for different components on the BIG-IP node in terms of the network state of the BIG-IP nodes like interfaces and vlans  as well as relating to the components responsible for application delivery like virtual servers and pools.

## Getting Ready

To follow along with this recipe, an ansible inventory is assumed to be already setup and IP connectivity between Ansible and the BIG-IP nodes is already established with the correct user credentials.

## How to do it..

- Create a new ansible playbook pb_f5_validate.yml with the below contents.

---

- name: Validating BIG-IP Health

hosts: ltm01

connection: local

tasks:

- name: Collect Device Facts from BIG-IP

bigip_device_facts:

gather_subset:

- interfaces

provider: "{{ conn_parameters }}"

register: bigip_facts

- Update the playbook pb_f5_validate.yml with a new task to validate interface status

- name: Validate All Interface are operational

assert:

that:

- item.enabled == 'yes'

fail_msg: " Interface {{ item.name }} is Down"

loop: "{{bigip_facts.interfaces}}"

## How it works..

The REST API supported on the BIG-IP Node is also providing interface to retrieve operational data from the device and it output all this data in JSON format. The below snippet outline the Interface status gathered from the BIG-IP nodes using the bigip_device_facts.

"bigip_facts": {
< -- Output Omitted for brevity -->

"interfaces": [

{

"active_media_type": "10000T-FD",

"bundle": "not-supported",

"bundle_speed": "not-supported",

"enabled": "yes",

"flow_control": "tx-rx",

"full_path": "1.1",

"if_index": 48,

"lldp_admin": "txonly",

"mac_address": "00:50:00:00:01:01",

"media_sfp": "auto",

"mtu": 1500,

"name": "1.1",
< -- Output Omitted for brevity -->

}

We retrieve the operational facts from the BIG-IP nodes using bigip_device_facts and we restrict only the data retrieved from the node using gather_subset and we only include the interfaces option to get only interface data. We save all the retrieved output to the variable bigip_facts.

We use the assert module to validate that all the interface are enabled and operational from the retrieved data and we loop over the bigip_facts variable (which is a list) to loop over all the interface and confirm that they are enabled.

## There is More..

In case we need to get the operational data for the application that we have deployed on the LTM node, we create a new playbook to validate the application deployment as shown below using the bigip_device_facts and in this time we limit the data retrieved for only the virtual-servers and we validate the data using the assert statement as done in the previous playbook. Below is the playbook contents for application deployment.

---

- name: Validating BIG-IP Health

hosts: ltm01

connection: local

vars_files: web_app.yml

tasks:

- name: Collect Device Facts from BIG-IP

bigip_device_facts:

gather_subset:

- virtual-servers

provider: "{{ conn_parameters }}"

register: bigip_app_facts

- name: Validate Virtual Address Status

assert:

that:

- item.enabled == 'yes'

- item.destination_address == vip

- item.destination_port == vip_port

fail_msg: " {{ item.name }} is No Setup Correctly"

loop: "{{bigip_app_facts.virtual_servers}}"

For sure we can enhance the validation to validate on multiple items on the virtual-address data as well as validating multiple different facts like ltm-pools to build a more comprehensive validation for the application deployed.

## See Also..

For more information regarding the Ansible bigip_device_facts module and all the information that we can retrieve from the BIG-IP node please check the below URL

https://docs.ansible.com/ansible/latest/modules/bigip_device_facts_module.html#bigip-device-facts-module